Timing software for use with Deltronics interfaces

User Guide

Church Road Industrial Estate, Gorslas,

Llanelli, Carmarthenshire, SA14 7NN

Tel: 01269 843728 Fax: 01269 845527

www.deltronics.co.uk

Introduction

This program allows measurement of Time, Velocity and Acceleration using any Deltronics interface having A and B timing inputs. (i.e. Extra Sense, Serial Interface, Control It Plus) It is also compatible with the Commotion Serial Control Box.

NOTE: It will not work with MiniSense or the Junior Serial Interface as these interfaces have no timing inputs.

Momentum and Kinetic Energy can also be derived.

There are many applications for this software – measuring acceleration due to gravity using a falling object or a trolley on a slope, investigation of the conservation of momentum, the laws of friction and the concepts of potential and kinetic energy.

Measurements are made by connecting one or two light gates to the A and B inputs. The software measures the time taken for a card of  known width to pass through. The card is attached to the object of interest e.g. a trolley on a slope.

The results are displayed as a table of values in the relevant units, along with the average value which is updated every time a new reading is taken.

The table can be saved in a CSV format which is readable by spreadsheets. it can also be copied and pasted to a spreadsheet or printed.

# Setting up the interface

Select Interface... from the Setup menu, select the appropriate interface and COM port and click OK. To save the settings for future sessions, tick the Save Settings box.

# Making Measurements

Use the radio buttons to select the type of measurement to be made -

Time, Velocity, Acceleration, Momentum or Kinetic Energy.

Select the measurement type using the other set of radio buttons -

At A, from A to B or At A or B

Set up the Parameters - Select Parameters... from the Setup menu

##### The Parameters Dialogue

You must put in at least the value of the card length (in metres).

If your measurement is from A to B then you must also enter the distance from A to B (in metres)

and if you are measuring momentum or Kinetic Energy then you must also enter the mass of the object (in Kg)

The values of card length, distance and mass are always displayed in the panel at the bottom of the window.

You can also select the number of decimal places (1,2,3 or 4) to be displayed, by selecting from the drop-down list.

### All values are in metres or Kilograms

To start measurements, click the START button. The A and B icons in the bottom right corner will reflect the state of the light gates connected to the A and B inputs.

The panel above will display 'Timing' The values of the measurements are added to the Table in the Table window as they occur. The average value is displayed in the window below this, and updated every time a new measurement is made.

If you make a 'bad measurement' e.g. if the object hits the side of the light gate, you can remove the last measurement by selecting Delete Last from the Edit menu.

Press STOP to stop measuring. The panel above the A and B icons changes to 'Stopped'

Pressing the Reset button will delete all the current entries in the table.

## How the measurements are made

The following section describes how the various quantities are measured and calculated. Knowing this will help you design your experiments to achieve the best possible accuracy.

The interface can only measure time; all other quantities are derived by calculation from a time measurement and from the information you supply about card length, object mass etc.

### Time

Time at A is how long the card takes to pass through the light gate connected to A.

Time from A to B is measured from when the leading edge of the card intercepts A to when the leading edge of the card intercepts B

Time at A or B measures the time at A and at B and displays the results in two separate columns.

#### Velocity

Velocity at A is the length of the card (input in the Parameters dialogue) divided by the time taken for the card to pass through A

Velocity from A to B is the distance from A to B(input in the Parameters dialogue) divided by the time measured from when the leading edge of the card intercepts A to when the leading edge of the card intercepts B

Velocity at A or B measures the velocity at A and at B and displays the results in two separate columns.

#### Acceleration

Acceleration at A is measured using a special card with two sections (see picture in main window when Acceleration at A is selected)

The two sections must be of equal width  (this is the card segment length parameter input in the Parameters dialogue)

The measurement is the velocity of the second section minus the velocity of the first section divided by the time elapsed between when the centre of the first section is at A and when the centre of the second section is at A. (the accuracy of this method relies on assuming that the acceleration is constant i.e. da/dt = 0)

Acceleration from A to B is the velocity at B minus the velocity at A divided by the time elapsed between when the centre of the card is at A to when the centre of the card is at B.

Again it is assumed that acceleration is constant.

Acceleration at A or B measures the acceleration at A and at B and displays the results in two separate columns.

#### Momentum

Momentum is calculated from velocity using the expression mv where m is the mass of the object (input in the Parameters dialogue) and v is the velocity.

### Kinetic Energy

Kinetic energy is calculated from the velocity using the expression (1/2)mv2 where m is the mass of the object (input in the Parameters dialogue) and v is the velocity.

###### Units

All values are calculated and displayed in SI units

Quantity				Unit				Unit symbol

Time					Seconds			S

Velocity				metres/second			m/S or mS-1

Acceleration				metres/second/second		m/S/S  or mS-2

Momentum				Kilogram metres/second	Kgm/S  or KgmS-1

Kinetic Energy			Joules				J

Manipulating the data

Once the data is gathered you can save it as a .csv (comma separated values) file Select Save/Save as... from the File menu. The file can then be opened by most spreadsheet and database software e.g. Microsoft Works , Excel.

You can also select Copy Table from the Edit menu; this copies the whole table to the clipboard, so that it can be pasted into a spreadsheet.  You can select part of the table by clicking and dragging the mouse over the required part. Selecting Copy from the Edit menu then copies the selected section to the clipboard.

Selecting Open... from the File menu allows you to open a previously saved file, subsequent measurements can then be added to it.

Select Print... from the File menu to print the current table to the printer. (select Print Setup... from the File menu to select a different printer or change printer settings).