VIRTUAL MODELS

By

Paul Johansson and Neil Lloyd

Computer Control programming

activities suitable for

KS3 and KS4 Technology

Photocopiable resources for use within schools with a ‘Virtual Models’ option for Control It PC site licence.

# Virtual Models

Virtual Models are designed to help you to understand how the control software will operate when connected to a real interface and real model. They include amusing characters like Coco the clown and develop to real life applications such as a pelican crossing. For each activity, think carefully about how to make the model work in a realistic way. You will need to work out the binary numbers and decimal output numbers for the more complex models.

Loading a virtual model

Several virtual models are supplied on  the disk.

To Load a model, follow these steps

1. Click File in the Procedures window

2. Click Open Model…

3. Click on the model of your choice eg Clown

4. Click Open

The model appears in a window. you can hide the Virtual Model window by selecting Model from the Window menu in the Control Window. To show it again, select Model again.

The 'LEDs' on the model will correspond to the outputs on the screen and on the box.

Programming in Control It PC

Repeats and Loops

Repeat loops are very useful elements in building most procedures.

This procedure repeats 6 times

Repeat 6

Switch on 1

Wait 10

Switch off 1

Wait 10

Next

The next procedure will continue to repeat until you click STOP

Repeat forever

Switch on 2

Wait 10

Switch off 2

Wait 10

Next

REPEAT can be used with UNTIL to check if an INPUT has changed

(Connect a switch to input 1 on the interface)

Repeat

Switch on 2

Wait 10

Switch off 2

Wait 10

Until Input 1 ON

When the input (switch) is ON, the procedure will stop repeating.  You can also click the INPUT LED to simulate the input switching ON.

Programming Tricks

Procedures can run procedures:	Create the following:

Build CAT

Repeat 4 Switch On All Wait 5 Switch Off All Wait 5 Next Next

Build DOG

Repeat 5 Output 12 Wait 5 Output 3 Wait 5 Next Next

Build MOUSE

Repeat 5 Switch On 2,4 Wait 5 Switch On 1,3 Wait 5 Output 0 Next Next

Now link them all together by creating a program called ANIMALS

Build ANIMALS

Repeat 10 CAT DOG MOUSE Next Next

By breaking a control program into smaller programs, it is much easier to understand and create your own control systems.  It also saves a lot of time and effort re-typing codes into the computer.

Making Inputs Work

Making a model, that you have made, respond to an input or a switch being pressed is quite tricky.  The system has to check continually to see if an input has changed.

The diagram below shows a flowchart which would make a bulb flash when  a light sensor is covered.

The procedure to do this would look like this:

| Build START |  | Build LIGHT |
| --- | --- | --- |
| Repeat Forever If Input 1 On Then Light Next | => <= | Switch On All Wait 3 Switch Off All Wait 3 |

When the INPUT is ON the procedure LIGHT will run.  If the INPUT remains on it will continue to run LIGHT.

When the INPUT IS OFF the procedure START will be stuck in a REPEAT FOREVER loop.

Programming Tricks

Variables

Create the following procedure:

Build LIGHTS

Repeat Forever Let A = 0 Repeat 4 Switch On A Wait 1 Switch Off A Let A = A + 1 Next Next

Now extend this procedure to make Coco juggle backwards as well using:

Let A = A - 1

Programming Tricks

You can have more fun with VARIABLES and using INPUT channels 1 and 2.  Key in this procedure and click on INPUTS 1 and 2 to test it.  Try involving another INPUT channel to create a third pattern,

You are now a computer control programmer!

JUGGLE

Repeat Forever Let A = 0 Repeat Switch On A Wait 1 Switch Off A Let A = A + 1 ## If A = 8 Then Let A = 0 Until Input 1 On Let A = 7 Repeat Switch On A Wait 1 Switch Off A Let A = A - 1 If A = -1 Then Let A = 7 Until Input 2 On Next

Programming Tricks

COUNTING

Make the computer count from 0 to 15.  This is the hard way, it means typing 15 lines of program:

build DAFT

OUTPUT 1

OUTPUT 2

OUTPUT 3

---------------

---------------

OUTPUT 15

Using a variable, counting is quite simple and easy.  The LEDs display binary numbers

Build COUNTER

Let A = 0 Repeat Output A Wait 2 Let A = A +1 Until A = 16

a)	Change your program to make the computer count in two's

b)	Change your program so that the computer counts up to 15 and 		then back down again to 0

(You will need to create two programs, perhaps call them UP and DOWN)